Quality

Layer height

- Layer height: 0.28

- First layer height: 0.28

Line width

- Default: 0.5

- First layer: 0.48

- Outer wall: 0.5

- Inner wall: 0.5

- Top surface: 0.45

- Sparse infill: 0.5

- Internal solid infill: 0.42

- Support: 0.42

Seam

- Seam position: Random

- Staggered inner seams: Disabled

- Seam gap: 10%

- Role base wipe speed: 100%

- Wipe speed: 200 mm/s

- Wipe on loops: Enabled (curved contours)

- Wipe before external loop: Enabled

Precision

- Slice gap closing radius: 0.02 mm

- Resolution: 0.012 mm

- Arc fitting: Enabled

- X-Y hole compensation: 0 mm

- X-Y contour compensation: 0 mm

- Elephant foot compensation: 0.05 mm

- Elephant foot compensation layers: 1

- Precise wall: Enabled

- Precise Z height: Disabled

- Convert holes to poly holes: Disabled

Ironing

- Ironing type: None

Wall generator

- Wall generator: Classic

Walls and surfaces

- Walls printing order: Inner/Outer

- Print infill first: Disabled

- Wall loop direction: Automatic

- Top surface flow ratio: 1

- Bottom surface flow ratio: 1

- Only one wall on top surface: Disabled

- Only one wall on first layer: Disabled

- Avoid crossing walls: Enabled

Bridging

- Bridge flow ratio: 1

- Internal bridge flow ratio: 1

- External bridge density: 100%

- Internal bridge density: 100%

- Thick external bridges: Disabled

- Filter out small internal bridges: Filter

- Bridge counterbore holes: None

Overhangs

- Detect overhang walls: Enabled

- Make overhang walls: Disabled

- Make overhangs printable: Disabled

- Extra perimeter on overhangs: Disabled

- Reverse on even: Disabled

Walls

- Wall loops: 4

- Alternate extra wall: Disabled

- Detect thin walls: Disabled

Top/bottom shells

- Top shell layers: 2

- Top shell thickness: 1 mm

- Top surface density: 100%

- Top surface pattern: Monotonic lines

- Bottom shell layers: 2

- Bottom shell thickness: 0 mm

- Bottom surface density: 100%

- Bottom surface pattern: Monotonic

- Top/Bottom solid infill/wall overlap: 25%

Infill

- Sparse infill density: 3%

- Fill multiline: 1

- Sparse infill pattern: Gyroid

- Sparse infill direction: 45°

- Internal solid infill: Monotonic

- Infill wall overlap: 15%

- Apply gap fill: None

- Anchor length: 400%

Support

- Enable support: Enabled

- Type: Tree (auto)

- Style: Default

- Overhang threshold angle: 60°

- Threshold overlap: 50%

- First layer density: 90%

- First layer expansion: 2 mm

- On build plate only: Disabled

- Only critical regions: Disabled

- Ignore small overhangs: Enabled

Advanced (Support)

- Top Z distance: 0.16 mm

- Bottom Z distance: 0.18 mm

- Base pattern: Rectilinear

- Base pattern spacing: 4.5 mm

- Pattern angle: 0°

- Top interface layers: 2

- Bottom interface layers: 2

- Top interface spacing: 0.4 mm

- Bottom interface spacing: 0.5 mm

- Support/object XY distance: 0.3 mm

- Support/object first layer gap: 0.2 mm

- Max bridge length without support: 10 mm

Tree support

- Tip diameter: 0.8 mm

- Branch distance: 4 mm

- Branch density: 30%

- Branch diameter: 2 mm

- Branch diameter angle: 5°

- Branch angle: 45°

- Preferred branch angle: 25°

Skirt

- Skirt loops: 1

- Skirt type: Combined

- Minimum extrusion length: 0 mm

- Skirt distance: 3 mm

- Skirt start point: -135°

- Skirt speed: 100 mm/s

- Skirt height: 1 layer

- Draft shield: Disabled

- Single loop after first layer: Disabled

Build Plate Adhesion

- Adhesion type: None

- Adhesion width: 0 mm

- Adhesion to object spacing: 0.1 mm

- Adhesion follows compensated outline: Disabled

Special Modes

- Slicing mode: Normal

- Print sequence: By layer

- Infill/wall order: Default

- Spiral vase: Disabled

Fuzzy Skin

- Fuzzy skin: Disabled